Report on Event 2 And 4 of Lab Component

# Computer Networks (EC620L)
Lab Report

| Name: | Mohamed Farhan Fazal |
| --- | --- |
| Roll No: | 27 |
| USN: | 01JST18EC055 |
| Division: | A |

Submitted to

Prof. Chandrashekhar Murthy B N
Assistant Professor
Dept. of ECE, JSS STU

2021

- SUBNETTING

Topology Diagram

Objectives

- Design and document an addressing scheme based on requirements.

- Select appropriate equipment and cable the devices.

- Apply a basic configuration to the devices.

- Configure static and default routing.

- Verify full connectivity between all devices in the topology.

This activity focuses on Subnetting skills, basic device configurations and static routing. Once you have configured all devices, you will test for end to end connectivity and examine your configuration.

Task 1: Design and document an addressing scheme.

Step 1: Design an addressing scheme.

Based on the network requirements shown in the topology, design an appropriate addressing scheme.

- The HQ, B1, B2, and B3 routers each have an address space. Subnet the address space basedon the hostrequirements.

- For each address space, assign subnet zero to the Fa0/0 LAN, subnet 1 to the Fa0/1, and soon.

Step 2: Document the addressing scheme.

- Document the IP addresses and subnet masks. Assign the first IP address to the routerinterface.

- For the WAN links, assign the first IP address toHQ.

Task 2: Apply a basic configuration.

Using your documentation, configure the routers with basic configurations including addressing and hostnames. Use 64000 as the clock rate. ISP is the DCE in its WAN link to HQ. HQ is the DCE for all other WAN links.

Task 3: Configure static and default routing

Configure static and default routing using the exit interface argument.

- HQ should have three static routes and one default route.

- B1, B2, and B3 should have one default route.

- ISP should have seven static routes. This will include the three WAN links between HQ and the branch routers B1, B2, and B3.

Task 5: Test connectivity and examine the configuration.

Step 1: Test connectivity.

- You should now have end-to-end connectivity. Use ping to test connectivity across the network. Each router should be able to ping all other router interfaces and the Web Server.

- Use extended ping to test LAN connectivity to the Web Server. For example, the test the Fa0/0 interface on B1, you would do the following:

- Troubleshoot until pings are successful.

Step 2: Examine the configuration.

Use verification commands to make sure your configurations are complete.

# Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| HQ | Fa0/0 | 192.168.0.129 | 255.255.255.192 |
|  | Fa0/1 | 192.168.0.129 | 255.255.255.192 |
|  | S0/0/0 | 10.0.0.1 | 255.255.255.252 |
|  | S0/0/1 | 10.0.0.5 | 255.255.255.252 |
|  | S0/1/0 | 10.0.0.9 | 255.255.255.252 |
|  | S0/1/1 | 209.165.201.2 | 255.255.255.252 |
| B1 | Fa0/0 | 192.168.1.1 | 255.255.255.192 |
|  | Fa0/1 | 192.168.1.65 | 255.255.255.192 |
|  | Fa1/0 | 192.168.1.129 | 255.255.255.192 |
|  | Fa1/1 | 192.168.1.193 | 255.255.255.192 |
|  | S0/0/0 | 10.0.0.2 | 255.255.255.252 |
| B2 | Fa0/0 | 172.16.0.1 | 255.255.252.0 |
|  | Fa0/1 | 172.16.4.1 | 255.255.252.0 |
|  | Fa1/0 | 172.16.8.1 | 255.255.252.0 |
|  | Fa1/1 | 172.16.12.1 | 255.255.252.0 |
|  | S0/0/0 | 10.0.0.6 | 255.255.255.252 |
| B3 | Fa0/0 | 172.20.0.1 | 255.255.224.0 |
|  | Fa0/1 | 172.20.32.1 | 255.255.224.0 |
|  | Fa1/0 | 172.20.64.1 | 255.255.224.0 |
|  | Fa1/1 | 172.20.128.1 | 255.255.224.0 |
|  | S0/0/0 | 10.0.0.10 | 255.255.255.252 |
| ISP | S0/0/0 | 209.165.201.1 | 255.255.255.252 |
|  | Fa0/0 | 209.165.200.225 | 255.255.255.252 |
| Web Server | NIC | 209.165.200.226 | 255.255.255.252 |

# Subnetting Scenario - VLSM

# Topology

100 Hosts

40 Hosts

2 Hosts

20Hosts

10Hosts

Objectives

- Design an IP Addressing Scheme(VLSM)

- Assign IP Addresses to Network Devices and Verify Connectivity

# Scenario

In this activity, you are given the network address of 195.152.92.0/24 to subnet and provide the IP addressing for the network shown in the topology. Each LAN in the network requires enough space for, atleast, 25 addresses for end devices, the switch and the router. The connection between R1 to R2 will require an IP address for each end of thelink.

# Design an IP Addressing Scheme

Step 1: Subnet the 195.152.92.0/24 network into the appropriate number of subnets.

- Based on the topology, how many subnets areneeded?

- How many bits must be borrowed to support the number of subnets in the topologytable?

- How many subnets does thiscreate?

- How many usable hosts does this create persubnet?

Note: If your answer is less than the 100 hosts required, then you borrowed too many bits.

- Calculate the binary value for the first fivesubnets.

Net 0: 195. 152. 92 .0 	0 0 0 0 0 0 0

Net 1: 195. 152. 92. 1 0 	   0 0 0 0 0 0

Net 2: 195. 152. 92. 1 1 0     0 0 0 0 0

Net 3: 195. 152. 92. 1 1 1 0     0 0 0 0

Net 4: 195. 152. 92. 1 1 1 1 0 0    0 0

- Calculate the binary and decimal value of the new subnetmask.

Subnet 0Mask:255.255.255.128	Subnet 3 Mask: 255.255.255.240

Subnet 1Mask:255.255.255.192	subnet 4 Mask: 255.255.255.252

Subnet 2 Mask:255.255.255.224

- Fill in the Subnet Table, listing the decimal value of all available subnets, the first and lastusable host address, and the broadcast address. Repeat until all addresses arelisted.

Note: You may not need to use all rows.

Subnet Table

| Sub net Num ber | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 195.152.92.0 | 195.152.92.1 | 195.152.92.126 | 195.152.92.127 |
| 1 | 195.152.92.128 | 195.152.92.129 | 195.152.92.190 | 195.152.92.191 |
| 2 | 195.152.92.192 | 195.152.92.193 | 195.152.92.221 | 195.152.92.223 |
| 3 | 195.152.92.224 | 195.152.92.225 | 195.152.92.238 | 195.152.92.239 |
| 4 | 195.152.92.240 | 195.152.92.241 | 195.152.92.242 | 195.152.92.243 |

# Step 02: Assign the subnets to the network shown in the topology.

- Assign Subnet 0 to the LAN connected to the GigabitEthernet 0/0 interface ofR1.

- Assign Subnet 1 to the LAN connected to the GigabitEthernet 0/1 interface ofR1.

- Assign Subnet 2 to the LAN connected to the GigabitEthernet 0/0 interface ofR2

- Assign Subnet 3 to the LAN connected to the GigabitEthernet 0/1 interface ofR2

- Assign Subnet 4 to the WAN link between R1 toR2

Step 3: Document the addressing scheme.

Fill in the Addressing Table using the following guidelines:

- Assign the first usable IP addresses to R1 for the two LAN links and the WANlink.

- Assign the first usable IP addresses to R2 for the LANs links. Assign the last usable IP address for the WANlink.

- Assign the second usable IP addresses to thehosts.

- Assign the last usable IP addresses to thehosts.

Addressing Table

| Devi ce | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 195.152.92.1 | 255.255.255.128 | N/A |
|  | G0/1 | 195.152.92.129 | 255.255.255.192 | N/A |
|  | S0/0/0 | 195.152.92.241 | 255.255.255.252 | N/A |
| R2 | G0/0 | 195.152.92.193 | 255.255.255.224 | N/A |
|  | G0/1 | 195.152.92.225 | 255.255.255.240 | N/A |
|  | S0/0/0 | 195.152.92.242 | 255.255.255.252 | N/A |
| PC1 | NIC | 195.152.92.2 | 255.255.255.128 | 195.152.92.1 |
| PC2 | NIC | 195.152.92.130 | 255.255.255.192 | 195.152.92.129 |
| PC3 | NIC | 195.152.92.194 | 255.255.255.224 | 195.152.92.193 |
| PC4 | NIC | 195.152.92.226 | 255.255.255.240 | 195.152.92.225 |

Assign IP Addresses to Network Devices and Verify Connectivity

Most of the IP addressing is already configured on this network. Implement the following steps to complete the addressing configuration.

Step 1: Configure IP addressing on R1 LAN interfaces.

Step 2: Configure IP addressing on S3, including the default gateway Step 3: Configure IP addressing on PC4, including the default gateway.

Step 4: Verify connectivity.

Verify connectivity from LAN to every other LAN. However, you should be able to ping every IP address listed in the Addressing Table.

# Note:

  - Take the screen shot of the results which is verified in Simulation Mode /Real mode and write the inference.

  - After the completion of the experiment, answer all the viva question given below in the labitself.

# Designing and Implementing a VLSM Addressing Scheme

.

## Topology

Objectives

- Examine Network Requirements

- Design the VLSM Address Scheme

- Cable and Configure the IPv4 Network

Background / Scenario

Variable Length Subnet Mask (VLSM) was designed to avoid wasting IP addresses. With VLSM, a network is subnetted and then re-subnetted. This process can be repeated multiple times to create subnets of various sizes based on the number of hosts required in each subnet. Effective use of VLSM requires address planning.

In this lab, use the 175.15.125.0/17 network address to develop an address scheme for the network displayed in the topology diagram. VLSM is used to meet the IPv4 addressing requirements. After you have designed the VLSM address scheme, you will configure the interfaces on the routers with the appropriate IP address information.

Part 1: Examine Network Requirements

In Part 1, you will examine the network requirements to develop a VLSM address scheme for the network displayed in the topology diagram using the 175.15.125.0/17 network address.

Step 1: Determine how many host addresses and subnets are available.

How many host addresses are available in a /17 network? 32760

What is the total number of host addresses needed in the topology diagram? 31500             How many subnets are needed in the network topology? 6

Step 2: Determine the largest subnet.

What is the subnet description (e.g. BR1 G0/1 LAN or BR1-HQ WAN link)? HQ G0/0

How many IP addresses are required in the largest subnet? 16000

What subnet mask can support that many host addresses? 255.255.63.0 (/18)

How many total host addresses can that subnet mask support? 16384

# Part 2: Design the VLSM Address Scheme

#### Step 1: Calculate the subnet information.

Use the information that you obtained in Part 1 to fill in the following table.

| Subnet Description | Number of | Network Address |  | First Host |  | Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
|  | Hosts Needed | /CIDR |  | Address |  | Address |
| HQ G0/0 | 16,000 | 175.15.0.0 | 175.15.0.1 |  | 175.15.63.255 |  |
| HQ G0/1 | 8,000 | 175.15.64.0 | 175.15.64.1 |  | 175.15.95.255 |  |
| BR1 G0/1 | 4,000 | 175.15.96.0 | 175.15.96.1 |  | 175.15.111.255 |  |
| BR1 G0/0 | 2,000 | 175.15.112.0 | 175.15.112.1 |  | 175.15.119.255 |  |
| BR2 G0/1 | 1,000 | 175.15.120.0 | 175.15.120.1 |  | 175.15.123.255 |  |
| BR2 G0/0 | 500 | 175.15.124.0 | 175.15.124.1 |  | 175.15.125.255 |  |
| HQ S0/0/0 – BR1 S0/0/0 | 2 | 195.152.92.0 | 195.152.92.1 |  | 195.152.92.255 |  |
| HQ S0/0/1 – BR2 S0/0/1 | 2 | 195.152.93.0 | 195.152.93.1 |  | 195.152.93.255 |  |
| BR1 S0/0/1 – BR2 S0/0/0 | 2 | 195.152.94.0 | 195.152.94.1 |  | 195.152.94.255 |  |

Step 2: Complete the device interface address table.

Assign the first host address in the subnet to the Ethernet interfaces. HQ should be given the first host address on the Serial links to BR1 and BR2. BR1 should be given the first host address for the serial link to BR2.

| Device |  | Interface |  | IP Address |  | Subnet Mask |  | Device Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ | G0/0 |  | 175.15.0.1 |  | 255.255.192.0 |  | 16,000 Host LAN |  |
|  | G0/1 |  | 175.15.64.0 |  | 255.255.224 |  | 8,000 Host LAN |  |
|  | S0/0/0 |  | 195.152.92.0 |  | 255.255.255.0 |  | BR1 S0/0/0 |  |
|  | S0/0/1 |  | 195.152.93.0 |  | 255.255.255.0 |  | BR2 S0/0/1 |  |
| BR1 | G0/1 |  | 175.15.96.0 |  | 255.255.240.0 |  | 2,000 Host LAN |  |
|  | G0/0 |  | 175.15.112.0 |  | 255.255.248 |  | 4,000 Host LAN |  |
|  | S0/0/0 |  | 195.152.92.1 |  | 255.255.255.0 |  | HQ S0/0/0 |  |
|  | S0/0/1 |  | 195.152.94.0 |  | 255.255.255.0 |  | BR2 S0/0/0 |  |
| BR2 | G0/0 |  | 175.15.124.0 |  | 255.255.254 |  | 500 Host LAN |  |
|  | G0/1 |  | 175.15.120.0 |  | 255.255.252 |  | 1,000 Host LAN |  |
|  | S0/0/0 |  | 195.152.94.1 |  | 255.255.255.0 |  | BR1 S0/0/1 |  |
|  | S0/0/1 |  | 195.152.93.1 |  | 255.255.255.0 |  | HQ S0/0/1 |  |

#### Step 1: Cable the network as shown in the topology.

Step 2: Configure basic settings on each router.

a. Assign the device name to the router.

#### Step 3: Configure the interfaces on each router.

- Assign an IP address and subnet mask to each interface using the table that you completed in Part 2.

- Configure an interface description for each interface.

- Set the clocking rate on all DCE serial interfaces to 128000.

HQ(config-if)# clock rate 128000

- Activate the interfaces.

Step 4: Save the configuration on all devices. Step 5: Test Connectivity.

- From HQ, ping BR1’s S0/0/0 interface address.

- From HQ, ping BR2’s S0/0/1 interface address.

- From BR1, ping BR2’s S0/0/0 interface address.

- Troubleshoot connectivity issues if pings were not successful..

- Basic Router Configuration:

## Network diagram

## Lab instructions

This lab will test your ability to configure basic settings such as hostname, motd banner, encrypted passwords, and terminal options on a Cisco Catalyst 2960 switch simulated in Packet Tracer 7 & above version.

- Use the local laptop connect to the switch console and configure the laptop with the right parameters for console access to the Cisco 2960 Catalyst switch.Configure Switch hostname as LOCAL-Router

- Configure the message of the day as "Unauthorized access is forbidden"

- Configure the password for privileged mode access as "cisco". The password must be md5 encrypted

- Configure password encryption on the switch using the global configuration command

- Configure CONSOLE access with the following settings :
- Login enabled
- Password : ciscoconsole
- History size : 15 commands
- Timeout : 6'45''
- Synchronous logging

- Configure TELNET access with the following settings :
- Login enabled
- Password : ciscotelnet
- History size : 15 commands
- Timeout : 8'20''
- Synchronous logging

- Configure the IP address of the switch as 192.168.1.2/24 and its default gateway IP (192.168.1.1).

- Test telnet connectivity from the Remote Laptop using the telnet client.

- RIP Configuration:

## Topology Diagram

## Learning Objectives

Upon completion of this lab, you will be able to:

Cable a network according to the Topology Diagram.

Erase the startup configuration and reload a router to the default state.

Perform basic configuration tasks on a router.

Configure and activate interfaces.

Configure RIP routing on all routers.

Verify RIP routing using show and debug commands.

Reconfigure the network to make it contiguous.

Observe automatic summarization at boundary router.

Gather information about RIP processing using the debug ip rip command.

Configure a static default route.

Propagate default routes to RIP neighbors.

Document the RIP configuration.

## Scenarios

Scenario A: Running RIPv1 on Classful Networks

Scenario B: Running RIPv1 with Subnets and Between Classful Networks

Scenario C: Running RIPv1 on a Stub Network.

## Scenario A: Running RIPv1 on Classful Networks

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.2.1 | 255.255.255.0 | N/A |
| R2 | Fa0/0 | 192.168.3.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.2.2 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.4.2 | 255.255.255.0 | N/A |
| R3 | Fa0/0 | 192.168.5.1 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.4.1 | 255.255.255.0 | N/A |
| PC1 | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | NIC | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |
| PC3 | NIC | 192.168.5.10 | 255.255.255.0 | 192.168.5.1 |

Task 1: Prepare the Network.

Step 1: Cable a network that is similar to the one in the Topology Diagram.

You can use any current router in your lab as long as it has the required interfaces shown in the topology.

Note: If you use 1700, 2500, or 2600 routers, the router outputs and interface descriptions will appear different.

Step 2: Clear any existing configurations on the routers.

Task 2: Perform Basic Router Configurations.

Perform basic configuration of the R1, R2, and R3 routers according to the following guidelines:

- Configure the router hostname.

- Disable DNS lookup.

- Configure an EXEC mode password.

- Configure a message-of-the-day banner.

- Configure a password for console connections.

- Configure a password for VTY connections.

Task 3: Configure and Activate Serial and Ethernet Addresses.

Step 1: Configure interfaces on R1, R2, and R3.

Configure the interfaces on the R1, R2, and R3 routers with the IP addresses from the table under the Topology Diagram.

Step 2: Verify IP addressing and interfaces.

Use the show ip interface brief command to verify that the IP addressing is correct and that the interfaces are active.

When you have finished, be sure to save the running configuration to the NVRAM of the router.

Step 3: Configure Ethernet interfaces of PC1, PC2, and PC3.

Configure the Ethernet interfaces of PC1, PC2, and PC3 with the IP addresses and default gateways from the table under the Topology Diagram.

Step 4: Test the PC configuration by pinging the default gateway from the PC.

Task 4: Configure RIP.

Step 1: Enable dynamic routing.

To enable a dynamic routing protocol, enter global configuration mode and use the router command.

Enter router ? at the global configuration prompt to a see a list of available routing protocols on your router.

To enable RIP, enter the command router rip in global configuration mode.

R1(config)#router rip

R1(config-router)#

Step 2: Enter classful network addresses.

Once you are in routing configuration mode, enter the classful network address for each directly connected network, using the network command.

R1(config-router)#network 192.168.1.0

R1(config-router)#network 192.168.2.0

R1(config-router)#

The network command:

Enables RIP on all interfaces that belong to this network. These interfaces will now both send and receive RIP updates.

Advertises this network in RIP routing updates sent to other routers every 30 seconds.

When you are finished with the RIP configuration, return to privileged EXEC mode and save the current configuration to NVRAM.

R1(config-router)#end

%SYS-5-CONFIG_I: Configured from console by console

R1#copy run start

Step 3: Configure RIP on the R2 router using the router rip and network commands.

R2(config)#router rip

R2(config-router)#network 192.168.2.0

R2(config-router)#network 192.168.3.0

R2(config-router)#network 192.168.4.0

R2(config-router)#end

%SYS-5-CONFIG_I: Configured from console by console

R2#copy run start

When you are finished with the RIP configuration, return to privileged EXEC mode and save the current configuration to NVRAM.

Step 4: Configure RIP on the R3 router using the router rip and network commands.

R3(config)#router rip

R3(config-router)#network 192.168.4.0

R3(config-router)#network 192.168.5.0

R3(config-router)#end

%SYS-5-CONFIG_I: Configured from console by console

R3# copy run start

When you are finished with the RIP configuration, return to privileged EXEC mode and save the current configuration to NVRAM.

Task 5: Verify RIP Routing.

Step 1: Use the show ip route command to verify that each router has all of the networks in the topology entered in the routing table.

Routes learned through RIP are coded with an R in the routing table. If the tables are not converged as shown here, troubleshoot your configuration. Did you verify that the configured interfaces are active? Did you configure RIP correctly? Return to Task 3 and Task 4 to review the steps necessary to achieve convergence.

R1#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

C    192.168.1.0/24 is directly connected, FastEthernet0/0

C    192.168.2.0/24 is directly connected, Serial0/0/0

R    192.168.3.0/24 [120/1] via 192.168.2.2, 00:00:04, Serial0/0/0

R    192.168.4.0/24 [120/1] via 192.168.2.2, 00:00:04, Serial0/0/0

R    192.168.5.0/24 [120/2] via 192.168.2.2, 00:00:04, Serial0/0/0

R1#

R2#show ip route

<Output omitted>

R    192.168.1.0/24 [120/1] via 192.168.2.1, 00:00:22, Serial0/0/0

C    192.168.2.0/24 is directly connected, Serial0/0/0

C    192.168.3.0/24 is directly connected, FastEthernet0/0

C    192.168.4.0/24 is directly connected, Serial0/0/1

R    192.168.5.0/24 [120/1] via 192.168.4.1, 00:00:23, Serial0/0/1

R2#

R3#show ip route

<Output omitted>

R    192.168.1.0/24 [120/2] via 192.168.4.2, 00:00:18, Serial0/0/1

R    192.168.2.0/24 [120/1] via 192.168.4.2, 00:00:18, Serial0/0/1

R    192.168.3.0/24 [120/1] via 192.168.4.2, 00:00:18, Serial0/0/1

C    192.168.4.0/24 is directly connected, Serial0/0/1

C    192.168.5.0/24 is directly connected, FastEthernet0/0

R3#

Step 2: Use the show ip protocols command to view information about the routing processes.

The show ip protocols command can be used to view information about the routing processes that are occurring on the router. This output can be used to verify most RIP parameters to confirm that:

RIP routing is configured

The correct interfaces send and receive RIP updates

The router advertises the correct networks

RIP neighbors are sending updates

R1#show ip protocols

Routing Protocol is "rip"

Sending updates every 30 seconds, next due in 16 seconds

Invalid after 180 seconds, hold down 180, flushed after 240

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Redistributing: rip

Default version control: send version 1, receive any version

Interface             Send  Recv  Triggered RIP  Key-chain

FastEthernet0/0       1     2 1

Serial0/0/0           1     2 1

Automatic network summarization is in effect

Maximum path: 4

Routing for Networks:

192.168.1.0

192.168.2.0

Passive Interface(s):

Routing Information Sources:

Gateway         Distance      Last Update

192.168.2.2          120

Distance: (default is 120)

R1#

R1 is indeed configured with RIP. R1 is sending and receiving RIP updates on FastEthernet0/0 and Serial0/0/0. R1 is advertising networks 192.168.1.0 and 192.168.2.0. R1 has one routing information source. R2 is sending R1 updates.

Step 3: Use the debug ip rip command to view the RIP messages being sent and received.

Rip updates are sent every 30 seconds so you may have to wait for debug information to be displayed.

R1#debug ip rip

R1#RIP: received v1 update from 192.168.2.2 on Serial0/0/0

192.168.3.0 in 1 hops

192.168.4.0 in 1 hops

192.168.5.0 in 2 hops

RIP: sending  v1 update to 255.255.255.255 via FastEthernet0/0 (192.168.1.1)

RIP: build update entries

network 192.168.2.0 metric 1

network 192.168.3.0 metric 2

network 192.168.4.0 metric 2

network 192.168.5.0 metric 3

RIP: sending  v1 update to 255.255.255.255 via Serial0/0/0 (192.168.2.1)

RIP: build update entries

network 192.168.1.0 metric 1

Step 4: Discontinue the debug output with the undebug all command.

R1#undebug all

All possible debugging has been turned off

## Scenario B: Running RIPv1 with Subnets between Classful Networks

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0 | 172.30.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 172.30.2.1 | 255.255.255.0 | N/A |
| R2 | Fa0/0 | 172.30.3.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 172.30.2.2 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.4.9 | 255.255.255.252 | N/A |
| R3 | Fa0/0 | 192.168.5.1 | 255.255.255.0 | N/A |
|  | S0/0/1 | 192.168.4.10 | 255.255.255.252 | N/A |
| PC1 | NIC | 172.30.1.10 | 255.255.255.0 | 172.30.1.1 |
| PC2 | NIC | 172.30.3.10 | 255.255.255.0 | 172.30.3.1 |
| PC3 | NIC | 192.168.5.10 | 255.255.255.0 | 192.168.5.1 |

Task 1: Make Changes between Scenario A and Scenario B

Step 1: Change the IP addressing on the interfaces as shown in the Topology Diagram and the Addressing Table.

Sometimes when changing the IP address on a serial interface, you may need to reset that interface by using the shutdown command, waiting for the LINK-5-CHANGED message, and then using the no shutdown command. This process will force the IOS to starting using the new IP address.

R1(config)#int s0/0/0

R1(config-if)#ip add 172.30.2.1 255.255.255.0

R1(config-if)#shutdown

%LINK-5-CHANGED: Interface Serial0/0/0, changed state to administratively down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/0, changed state to down

R1(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/0/0, changed state to up

R1(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/0, changed state to up

Step 2: Verify that routers are active.

After reconfiguring all the interfaces on all three routers, verify that all necessary interfaces are active with the show ip interface brief command.

Step 3: Remove the RIP configurations from each router.

Although you can remove the old network commands with the no version of the command, it is more efficient to simply remove RIP and start over. Remove the RIP configurations from each router with the no router rip global configuration command. This will remove all the RIP configuration commands including the network commands.

R1(config)#no router rip

R2(config)#no router rip

R3(config)#no router rip

Task 2: Configure RIP

Step 1: Configure RIP routing on R1 as shown below.

R1(config)#router rip

R1(config-router)#network 172.30.0.0

Notice that only a single network statement is needed for R1. This statement includes both interfaces on different subnets of the 172.30.0.0 major network.

Step 2: Configure R1 to stop sending updates out the FastEthernet0/0 interface.

Sending updates out this interface wastes the bandwidth and processing resources of all devices on the LAN. In addition, advertising updates on a broadcast network is a security risk. RIP updates can be intercepted with packet sniffing software. Routing updates can be modified and sent back to the router, corrupting the router table with false metrics that misdirects traffic.

The passive-interface fastethernet 0/0 command is used to disable sending RIPv1 updates out that interface. When you are finished with the RIP configuration, return to privileged EXEC mode and save the current configuration to NVRAM.

R1(config-router)#passive-interface fastethernet 0/0

R1(config-router)#end

%SYS-5-CONFIG_I: Configured from console by console

R1#copy run start

Step 3: Configure RIP routing on R2 as shown below.

R2(config)#router rip

R2(config-router)#network 172.30.0.0

R2(config-router)#network 192.168.4.0

R2(config-router)#passive-interface fastethernet 0/0

R2(config-router)#end

%SYS-5-CONFIG_I: Configured from console by console

R2#copy run start

Again notice that only a single network statement is needed for the two subnets of 172.30.0.0. This statement includes both interfaces, on different subnets, of the 172.30.0.0 major network. The network for the WAN link between R2 and R3 is also configured.

When you are finished with the RIP configuration, return to privileged EXEC mode and save the current configuration to NVRAM.

Step 4: Configure RIP routing on R3 as shown below.

R3(config)#router rip

R3(config-router)#network 192.168.4.0

R3(config-router)#network 192.168.5.0

R3(config-router)#passive-interface fastethernet 0/0

R3(config-router)#end

%SYS-5-CONFIG_I: Configured from console by console

R3#copy run start

When you are finished with the RIP configuration, return to privileged EXEC mode and save the current configuration to NVRAM.

Task 3: Verify RIP Routing

Step 1: Use the show ip route command to verify that each router has all of the networks in the topology in the routing table.

R1#show ip route

<Output omitted>

172.30.0.0/24 is subnetted, 3 subnets

C       172.30.1.0 is directly connected, FastEthernet0/0

C       172.30.2.0 is directly connected, Serial0/0/0

R       172.30.3.0 [120/1] via 172.30.2.2, 00:00:22, Serial0/0/0

R    192.168.4.0/24 [120/1] via 172.30.2.2, 00:00:22, Serial0/0/0

R    192.168.5.0/24 [120/2] via 172.30.2.2, 00:00:22, Serial0/0/0

R1#

R2#show ip route

<Output omitted>

172.30.0.0/24 is subnetted, 3 subnets

R       172.30.1.0 [120/1] via 172.30.2.1, 00:00:04, Serial0/0/0

C       172.30.2.0 is directly connected, Serial0/0/0

C       172.30.3.0 is directly connected, FastEthernet0/0

192.168.4.0/30 is subnetted, 1 subnets

C       192.168.4.8 is directly connected, Serial0/0/1

R    192.168.5.0/24 [120/1] via 192.168.4.10, 00:00:19, Serial0/0/1

R2#

R3#show ip route

R    172.30.0.0/16 [120/1] via 192.168.4.9, 00:00:22, Serial0/0/1

192.168.4.0/30 is subnetted, 1 subnets

C       192.168.4.8 is directly connected, Serial0/0/1

C    192.168.5.0/24 is directly connected, FastEthernet0/0

Step 2: Verify that all necessary interfaces are active.

If one or more routing tables does not have a converged routing table, first make sure that all necessary interfaces are active with show ip interface brief.

Then use show ip protocols to verify the RIP configuration. Notice in the output from this command that the FastEthernet0/0 interface is no longer listed under Interface but is now listed under a new section of the output: Passive Interface(s).

R1#show ip protocols

Routing Protocol is "rip"

Sending updates every 30 seconds, next due in 20 seconds

Invalid after 180 seconds, hold down 180, flushed after 240

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Redistributing: rip

Default version control: send version 2, receive version 2

Interface             Send  Recv  Triggered RIP  Key-chain

Serial0/1/0           2     2

Automatic network summarization is in effect

Maximum path: 4

Routing for Networks:

172.30.0.0

209.165.200.0

Passive Interface(s):

FastEthernet0/0

Routing Information Sources:

Gateway         Distance      Last Update

209.165.200.229      120      00:00:15

Distance: (default is 120)

Step 3: View the RIP messages being sent and received.

To view the RIP messages being sent and received use the debug ip rip command. Notice that RIP updates are not sent out of the fa0/0 interface because of the passive-interface fastethernet 0/0 command.

R1#debug ip rip

R1#RIP: sending  v1 update to 255.255.255.255 via Serial0/0/0 (172.30.2.1)

RIP: build update entries

network 172.30.1.0 metric 1

RIP: received v1 update from 172.30.2.2 on Serial0/0/0

172.30.3.0 in 1 hops

Step 4: Discontinue the debug output with the undebug all command.

R1#undebug all

All possible debugging has been turned off

## Scenario C: Running RIPv1 on a Stub Network

## Topology Diagram

## Background

In this scenario we will modify Scenario B to only run RIP between R1 and R2. Scenario C is a typical configuration for most companies connecting a stub network to a central headquarters router or an ISP. Typically, a company runs a dynamic routing protocol (RIPv1 in our case) within the local network but finds it unnecessary to run a dynamic routing protocol between the company’s gateway router and the ISP. For example, colleges with multiple campuses often run a dynamic routing protocol between campuses but use default routing to the ISP for access to the Internet. In some cases, remote campuses may even use default routing to the main campus, choosing to use dynamic routing only locally.

To keep our example simple, for Scenario C, we left the addressing intact from Scenario B. Let’s assume that R3 is the ISP for our Company XYZ, which consists of the R1 and R2 routers using the 172.30.0.0/16 major network, subnetted with a /24 mask. Company XYZ is a stub network, meaning that there is only one way in and one way out of the 172.30.0.0/16 network—in via R2 (the gateway router) and out via R3 (the ISP). It doesn’t make sense for R2 to send R3 RIP updates for the 172.30.0.0 network every 30 seconds, because R3 has no other way to get to 172.30.0.0 except through R2. It makes more sense for R3 to have a static route configured for the 172.30.0.0/16 network pointing to R2.

How about traffic from Company XYZ toward the Internet? It makes no sense for R3 to send over 120,000 summarized Internet routes to R2. All R2 needs to know is that if a packet is not destined for a host on the 172.30.0.0 network, then it should send the packet to the ISP, R3. This is the same for all other Company XYZ routers (only R1 in our case). They should send all traffic not destined for the 172.30.0.0 network to R2. R2 would then forward the traffic to R3.

Task 1: Make Changes between Scenario B and Scenario C.

Step 1: Remove network 192.168.4.0 from the RIP configuration for R2.

Remove network 192.168.4.0 from the RIP configuration for R2, because no updates will be sent between R2 and R3 and we don’t want to advertise the 192.168.4.0 network to R1.

R2(config)#router rip

R2(config-router)#no network 192.168.4.0

Step 2: Completely remove RIP routing from R3.

R3(config)#no router rip

Task 2: Configure the Static Route on R3 for the 172.30.0.0/16 network.

Because R3 and R2 are not exchanging RIP updates, we need to configure a static route on R3 for the 172.30.0.0/16 network. This will send all 172.30.0.0/16 traffic to R2.

R3(config)#ip route 172.30.0.0 255.255.252.0 serial0/0/1

Task 3: Configure a Default Static Route on R2.

Step 1: Configure R2 to send default traffic to R3.

Configure a default static route on R2 that will send all default traffic—packets with destination IP addresses that do not match a specific route in the routing table—to R3.

R2(config)# ip route 0.0.0.0 0.0.0.0 serial 0/0/1

Step 2: Configure R2 to send default static route information to R1.

The default-information originate command is used to configure R2 to include the default static route with its RIP updates. Configure this command on R2 so that the default static route information is sent to R1.

R2(config)#router rip

R2(config-router)#default-information originate

R2(config-router)#

Note: Sometimes it is necessary to clear the RIP routing process before the default-information originate command will work. First, try the command clear ip route * on both R1 and R2. This command will cause the routers to immediately flush routes in the routing table and request updates from each other. Sometimes this does not work with RIP. If the default route information is still not sent to R1, save the configuration on R1 and R2 and then reload both routers. Doing this will reset the hardware and both routers will restart the RIP routing process.

Task 4: Verify RIP Routing.

Step 1: Use the show ip route command to view the routing table on R2 and R1.

R2#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is 0.0.0.0 to network 0.0.0.0

172.30.0.0/24 is subnetted, 3 subnets

C       172.30.2.0 is directly connected, Serial0/0/0

C       172.30.3.0 is directly connected, FastEthernet0/0

R       172.30.1.0 [120/1] via 172.30.2.1, 00:00:16, Serial0/0/0

192.168.4.0/30 is subnetted, 1 subnets

C       192.168.4.8 is directly connected, Serial0/0/1

S*   0.0.0.0/0 is directly connected, Serial0/0/1

Notice that R2 now has a static route tagged as a candidate default.

R1#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is 172.30.2.2 to network 0.0.0.0

172.30.0.0/24 is subnetted, 3 subnets

C       172.30.2.0 is directly connected, Serial0/0/0

R       172.30.3.0 [120/1] via 172.30.2.2, 00:00:05, Serial0/0/0

C       172.30.1.0 is directly connected, FastEthernet0/0

R*   0.0.0.0/0 [120/1] via 172.30.2.2, 00:00:19, Serial0/0/0

Notice that R1 now has a RIP route tagged as a candidate default route. The route is the “quad-zero” default route sent by R2. R1 will now send default traffic to the Gateway of last resort at 172.30.2.2, which is the IP address of R2.

Step 2: View the RIP updates that are sent and received on R1 with the debug ip rip command.

R1#debug ip rip

RIP protocol debugging is on

R1#RIP: sending  v1 update to 255.255.255.255 via Serial0/0/0 (172.30.2.1)

RIP: build update entries

network 172.30.1.0 metric 1

RIP: received v1 update from 172.30.2.2 on Serial0/0/0

0.0.0.0 in 1 hops

172.30.3.0 in 1 hops

Notice that R1 is receiving the default route from R2.

Step 3: Discontinue the debug output with the undebug all command.

R1#undebug all

All possible debugging has been turned off

Step 4: Use the show ip route command to view the routing table on R3.

R3#show ip route

S    172.30.0.0/16 is directly connected, Serial0/0/1

192.168.4.0/30 is subnetted, 1 subnets

C       192.168.4.8 is directly connected, Serial0/0/1

C    192.168.5.0/24 is directly connected, FastEthernet0/0

Notice that RIP is not being used on R3. The only route that is not directly connected is the static route.

Task 5: Document the Router Configurations

On each router, capture the following command output to a text file and save for future reference:

Running configuration

Routing table

Interface summarization

Output from show ip protocols

  - Network Address Translation (NAT)

Objective

In this lab, static Network Address Translation (NAT) and dynamic NAT are configured.

Scenario

The network needs approximately 100 private IP addresses translated in a one-to-one fashion  with a pool of public IP addresses. To do this, network will use NAT translation with a portion of its  class C address space allocated by ISP1.

Step 1

Build and configure the network according to the diagram.

Use ping to test connectivity between the NAT and ISP1 routers, between the workstations and  the default gateway, and between WebServer and ISP1.

Step 2

Since no routing protocol will be enabled, configure a default route to the Internet on the NAT router: NAT(config)#ip route 0.0.0.0 0.0.0.0 200.200.100.2

ISP1 needs to be able to reach hosts on the 192.168.0/24 network. But these hosts will have their IP  addresses translated to public IP addresses in the 200.200.100.128/25 network, so a static route to  the 200.200.100.128/25 network is required:

ISP1(config)#ip route 200.200.100.128 255.255.255.128 200.200.100.1

Step 3

Create a standard Access Control List that defines all Internal Users:

NAT(config)#access-list 1 permit 192.168.1.0 0.0.0.255

Step 4

In this step, configure private and public address spaces to be used for NAT and configure  the translation:

The public address space 200.200.100.128/25 will be used as a pool to provide NAT translation for  the private IP addresses. To statically map the Internal User with IP address 192.168.1.2 pictured in  the diagram, enter the following command

NAT(config)#ip nat inside source static 192.168.1.2 200.200.100.252

This static mapping has the advantage of allowing “external” users to always access the host  192.168.1.2 by way of the fixed IP address 200.200.100.252 (in addition to letting the 192.168.1.2  Internal User access the Internet). On the down side, this external accessibility is also viewed as a  security vulnerability. To allow the other hosts on the internal (private) network to reach the Internet,  translations will need to be made for those hosts as well. A list of static translations could be made  one by one, but a simpler alternative is to configure a pool of addresses and let the router make  one-to-one dynamic NAT translations for these hosts. For example, to map the non-statically  mapped hosts in the 192.168.1.0/24 network to public IP addresses in the range 200.200.100.129  to200.200.100.250, proceed as follows:

NAT(config)#ip nat pool public 200.200.100.129 200.200.100.250

netmask 255.255.255.128

NAT(config)#ip nat inside source list 1 pool public

This provides a dynamic one-to-one NAT translation between public IP addresses in the “public” pool and  private IP addresses specified by access list 1. The Internal Users IP addresses are configured  independently of the NAT translation. Dynamic NAT translations are made for any internal hosts for  which no static translation has been defined. The configuration above reserves IP addresses 200.200.100.251 to 200.200.100.254 for use in further static NAT mappings. Static translations  are often used with an internal server to enable external access to it by way of a fixed external IP.

Note: If there are more than 128 active hosts on the private network, static NAT translation and/or dynamic one-to-one NAT translations will prevent more than 128 hosts from  accessing the Internet. For these additional hosts to get on the Internet, “NAT overloading”  must be configured (see Lab 2.10.4b).

Step 5

Now, designate the inside NAT interface and the outside NAT interface. In more complex  topologies, it is possible to have more than one inside NAT interface.

NAT(config)#interface fastethernet 0/0

NAT(config-if)#ip nat inside

NAT(config-if)#interface serial 0/0

NAT(config-if)#ip nat outside

There are several show commands that can be used to see if NAT is working: show ip nat translations, show ip nat statistics, and show ip nat translations verbose.

From the two Internal User workstations, ping WebServer (200.200.50.2). Then check that  WebServer is accessible by connecting from an Internal User workstation using a browser with the  WebServer IP address, 200.200.50.2. Issue the three NAT show commands listed above on the  NAT router. Sample outputs are shown below.

NAT#show ip nat translations

Pro Inside global Inside local Outside local Outside global --- 200.200.100.129 192.168.1.5 --- ---

--- 200.200.100.252 192.168.1.2 --- ---

NAT#show ip nat statistics

Total active translations: 2 (1 static, 1 dynamic; 0 extended)

Outside interfaces:

Serial0/0

Inside interfaces:

FastEthernet0/0

Hits: 131 Misses: 9

Expired translations: 0

Dynamic mappings:

-- Inside Source

[Id: 2] access-list 1 pool public refcount 1

pool public: netmask 255.255.255.128

start 200.200.100.129 end 200.200.100.250

type generic, total addresses 122, allocated 1 (0%), misses 0

NAT#show ip nat translations verbose

Pro Inside global Inside local Outside local Outside global --- 200.200.100.129 192.168.1.5 --- ---

create 00:02:55, use 00:02:55, left 23:57:04, Map-Id(In): 2,

flags:

none, use_count: 0

--- 200.200.100.252 192.168.1.2 --- ---

create 00:40:36, use 00:02:59,

flags:

static, use_count: 0

Notice that the Internal User with IP address 192.168.1.5 had its address dynamically translated  to 200.200.100.129, the first available address in the “public” pool. The command clear ip  nat translation * can be used to clear all dynamic NAT translations:

NAT#clear ip nat translation *

NAT#show ip nat translations

Pro Inside global Inside local Outside local Outside global --- 200.200.100.252 192.168.1.2 --- ---

Save the configurations for NAT and ISP1.

  - Verifying NAT and PAT Configuration

Objective

• Configure a router for Network Address Translation (NAT) and Port Address Translation (PAT) • Test the configuration and verify NAT/PAT statistics

Background/Preparation

The ISP has allocated a company the public CIDR IP address 199.99.9.32/30. This is equivalent  to four public IP addresses. Since the company has an internal requirement for more than 30  addresses, the IT manager has decided to use NAT with PAT. Routing between the ISP and the  gateway router is done using a static route between the ISP and the gateway, and a default route  between the gateway and the ISP. The ISP connection to the Internet will be represented by a  loopback address on the ISP router.

Cable a network similar to the one in the diagram above. Any router that meets the interface  requirements displayed on the above diagram may be used. This includes the following and any  of their possible combinations:

• 800 series routers

• 1600 series routers

• 1700 series routers

• 2500 series routers

• 2600 series routers

.

Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be  used based on the equipment in the lab. The configuration output used in this lab is produced  from 1721 series routers. Any other router used may produce slightly different output. Conduct the  following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Refer to the erase and reload instructions at the end of this lab. Perform those steps on  all routers in this lab assignment before continuing.

Step 1 Configure the routers

Configure all of the following according to the chart:

• The hostname

• The console

• The virtual terminal

• The enable passwords

• The interfaces

If problems occur during this configuration, refer to Lab 1.1.4a Configuring NAT.

Step 2 Save the configuration

At the privileged EXEC mode prompt, on both routers, type the command copy running config startup-config.

Step 3 Configure the hosts with the proper IP address, subnet mask, and default gateway

Each workstation should be able to ping the attached router. If for some reason this is not the case,  troubleshoot as necessary. Check and verify that the workstation has been assigned a specific IP  address and default gateway. If running Windows 98, check using Start > Run > winipcfg. If  running Windows 2000 or higher, check using ipconfig in a DOS window.

Step 4 Verify that the network is functioning

a. From the attached hosts, ping the FastEthernet interface of the default gateway router. b. Was the ping from the first host successful? - YES

c. Was the ping from the second host successful? - YES

d. If the answer is no for either question, troubleshoot the router and host configurations to find  the error. Then ping again until they both are successful.

Step 5 Create a static route

a. Create a static route from the ISP to the Gateway router. Addresses 199.99.9.32/27 have been  allocated for Internet access outside of the company. Use the ip route command to create  the static route.

ISP(config)#ip route 199.99.9.32 255.255.224.0 200.2.2.18

Step 6 Create a default route

a. Add a default route, using the ip route command, from the Gateway router to the ISP router.  This will forward any unknown destination address traffic to the ISP:

Gateway(config)#ip route 0.0.0.0 0.0.0.0 200.2.2.17

Step 7 Define the pool of usable public IP addresses

To define the pool of public addresses, use the ip nat pool command:

Gateway(config)#ip nat pool public-access 199.99.9.32

199.99.9.35 netmask 255.255.255.252

Step 8 Define an access list that will match the inside private IP addresses To define the access list to match the inside private addresses, use the access list command:

Gateway(config)#access-list 1 permit 10.10.10.0 0.0.0.255

Step 9 Define the NAT translation from inside list to outside pool

To define the NAT translation, use the ip nat inside source command:

Gateway(config)#ip nat inside source list 1 pool public-access overload

Step 10 Specify the interfaces

The active interfaces on the router need to be identified as either inside or outside interfaces  with respect to NAT. To do this, use the ip nat inside or ip nat outside command:

Gateway(config)#interface fastethernet 0

Gateway(config-if)#ip nat inside

Gateway(config-if)#interface serial 0

Gateway(config-if)#ip nat outside

Step 11 Testing the configuration

a. From the workstations, ping 172.16.1.1. Open multiple DOS windows on each  workstation and Telnet to the 172.16.1.1 address. Next, view the NAT translations on the  Gateway router, with the command show ip nat translations.

Step 12 Verify NAT and PAT Statistics

a. To view the NAT and PAT statistics type the show ip nat statistics command at the  privileged EXEC mode prompt.

Upon completion of the previous steps finish the lab by doing the following:

• Logoff by typing exit

• Turn the router off

• Remove and store the cables and adapter

Configuration reference sheet

This sheet contains the basic configuration commands for the ISP and Gateway routers:

ISP

Router#configure terminal

Router(config)#hostname ISP

ISP(config)#enable password cisco

ISP(config)#enable secret class

ISP(config)#line console 0

ISP(config-line)#password cisco

ISP(config-line)#login

ISP(config-line)#exit

ISP(config)#line vty 0 4

ISP(config-line)#password cisco

ISP(config-line)#login

ISP(config-line)#exit

ISP(config)#interface loopback 0

ISP(config-if)#ip address 172.16.1.1 255.255.255.255 ISP(config-if)#no shutdown

ISP(config-if)#exit

ISP(config)#interface serial 0

ISP(config-if)#ip address 200.2.2.17 255.255.255.252 ISP(config-if)#no shutdown

ISP(config-if)#clockrate 64000

ISP(config)#ip route 199.99.9.32 255.255.255.224

200.2.2.18 ISP(config)#end

ISP#copy running-config startup-config

Gateway

Router#configure terminal

Router(config)#hostname Gateway

Gateway(config)#enable password cisco

Gateway(config)#enable secret class

Gateway(config)#line console 0

Gateway(config-line)#password cisco

Gateway(config-line)#login

Gateway(config-line)#exit

Gateway(config)#line vty 0 4

Gateway(config-line)#password cisco

Gateway(config-line)#login

Gateway(config-line)#exit

Gateway(config)#interface fastethernet 0

Gateway(config-if)#ip address 10.10.10.1 255.255.255.0 Gateway(config-if)#no shutdown

Gateway(config-if)#exit

Gateway(config)#interface serial 0

Gateway(config-if)#ip address 200.2.2.18 255.255.255.252 Gateway(config-if)#no shutdown

Gateway(config)#ip route 0.0.0.0 0.0.0.0 200.2.2.17

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted for a password, enter class (if that does not work, ask the instructor). Router>enable

At the privileged EXEC mode, enter the command erase startup-config.

Router#erase startup-config

The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm.

The response should be:

Erase of nvram: complete

Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload

The responding line prompt will be:

System configuration has been modified. Save? [yes/no]:

Type n and then press Enter.

The responding line prompt will be:

Proceed with reload? [confirm]

Press Enter to confirm.

In the first line of the response will be:

Reload requested by console.

After the router has reloaded the line prompt will be:

Would you like to enter the initial configuration dialog? [yes/no]: Type n and then press Enter.

The responding line prompt will be:

Press RETURN to get started!

Press Enter.

Now the router is ready for the assigned lab to be performed.

Router Serial

| Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 |
| --- | --- | --- |
| Ethernet 0 (E0) | Ethernet 1 (E1) | NA |
| Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) |
| FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (S0) |
| Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) |
| FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) |

- Access Control List Basics and Extended Ping

Objective

This lab activity reviews the basics of standard and extended access lists.

Scenario

The LAN users connected to the Vista router are concerned about access to the network from  hosts on network 10.0.0.0. A standard access list must be used to block all access to the Vista  LAN from network 10.0.0.0 /24.

Also, an extended ACL must be used to block network 192.168.3.0 host access to Web servers  on the 10.0.0.0 /24 network.

Step 1

Build and configure the network according to the diagram. Use RIPv1, and enable updates on all  active interfaces with the appropriate network commands. The commands necessary to  configure SanJose1 are shown in the following example:

SanJose1(config)#router rip

SanJose1(config-router)#network 192.168.1.0

SanJose1(config-router)#network 10.0.0.0

Use the ping command to verify the work and test connectivity between all interfaces.

Step 2

Check the routing table on Vista using the show ip route command. Vista should have all  four networks in the routing table. Troubleshoot, if necessary

Access Control List Basics

Access Control Lists (ACLs) are simple but powerful tools. When the access list is configured,  each statement in the list is processed by the router in the order in which it was created. If an  individual packet meets a statement’s criteria, the permit or deny is applied to that packet, and no  further list entries are checked. Each packet starts at the top of the list, every time.

It is not possible to reorder an access list, skip statements, edit statements, or delete statements  from a numbered access list, while in the router configuration mode. With numbered access lists,  any attempt to delete a single statement results in deletion of the entire list. Named ACLs (NACLs)  do allow for the deletion of individual statements. It is suggested that ACLs, of all kinds, be created  in an off-line editor and pasted into the configuration.

The following concepts apply to both standard and extended access lists:

Two step process

The access list may be created with one or more access-list commands while in global  configuration mode. Second, the access list is applied to or referenced by other commands, such  as the ip access-group command which applies the ACL to an interface. An example would be  the following:

Vista#config terminal

Vista(config)#access-list 50 deny 10.0.0.0 0.0.0.255

Vista(config)#access-list 50 permit any

Vista(config)#interface fastethernet 0/0

Vista(config-if)#ip access-group 50 out

Vista(config-if)#^Z

Syntax and Keywords

The basic syntax for creating an access list entry is as follows:

router(config)#access-list # {permit | deny}ip address wildcard mask

The permit command allows packets matching the specified criteria to be accepted for  whatever application the access list is being used for. The deny command discards packets  matching the criteria on that line.

Two important keywords, any and host, can be used with IP addresses and the access list. The  keyword any matches all hosts on all networks, equivalent to 0.0.0.0 255.255.255.255. The  keyword host can be used with an IP address to indicate a single host address. The syntax is host ip address (host 192.168.1.10). This is the same as entering 192.168.1.10 0.0.0.0.

Implicit deny statement

Every access list contains a final “deny” statement that matches all packets. This is called the  implicit deny. Because the implicit deny statement is not visible in show command output, it is often  overlooked, with serious consequences. As an example, consider the following single-line access  list:

Router(config)#access-list 75 deny host 192.168.1.10

Access- list 75 clearly denies all traffic sourced from the host, 192.168.1.10. What might not be  obvious is that all other traffic will be discarded as well. This is because the implicit deny any is  the final statement in any access list.

At least one permit statement is required

There is no requirement that an ACL contain a deny statement. If nothing else, the implicit deny any statement takes care of that. But if there are no permit statements, the effect will be the  same as if there were only a single deny any statement.

Wildcard mask

In identifying IP addresses, ACLs use a wildcard mask instead of a subnet mask. Initially, the masks  might look the same, but closer observation reveals that they are very different. Remember that a binary  0 in a wildcard mask instructs the router to match the corresponding bit in the IP address. In/out

When deciding whether an ACL should be applied to inbound or outbound traffic, always view  things from the perspective of the router. In other words, determine whether traffic is coming into the  router, inbound, or leaving the router, outbound.

Applying ACLs

Extended ACLs should be applied as close to the source as possible, thereby conserving  network resources. Standard ACLs, by necessity, must be applied as close to the destination as  possible. This is because the standard ACL can match only at the source address of a packet.

Step 3

On the Vista router, create the following standard ACL and apply it to the LAN interface:

Vista#config terminal

Vista(config)#access-list 50 deny 10.0.0.0 0.0.0.255

Vista(config)#access-list 50 permit any

Vista(config)#interface fastethernet 0/0

Vista(config-if)#ip access-group 50 out

Vista(config-if)#^Z

Try pinging 192.168.3.2 from SanJose1.

The ping should be successful. This result might be surprising, because all traffic from the 10.0.0.0/8  network was just blocked. The ping is successful because, even though it came from SanJose1, it is  not sourced from the 10.0.0.0/8 network. A ping or traceroute from a router uses the closest  interface to the destination as the source address. Therefore, the ping is coming from the  192.168.1.0/24, SanJose1’s Serial 0/0.

SanJose1#ping 192.168.3.2

Sending 5, 100-byte ICMP Echos to 192.168.3.2, timeout is 2 seconds: !!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 4/4/4 ms

Step 4

In order to test the ACL from SanJose1, the extended ping command must be used to specify  a source interface as follows:

On SanJose1, issue the following commands:

SanJose1#ping

Protocol [ip]:

Target IP address: 192.168.3.2

Repeat count [5]:

Datagram size [100]:

Timeout in seconds [2]:

Extended commands [n]: y

Source address or interface: 10.0.0.1

Type of service [0]:

Set DF bit in IP header? [no]:

Validate reply data? [no]:

Data pattern [0xABCD]:

Loose, Strict, Record, Timestamp, Verbose[none]:

Sweep range of sizes [n]:

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.3.2, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

Step 5

Standard ACLs are numbered 1 - 99. IOS version 12.xx allows additional numbering from 1300 - 1699. Extended ACLs are numbered 100 - 199. IOS version 12.xx allows additional numbering  from 2000 - 2699. Extended ACLs can be used to enforce highly specific criteria for filtering  packets. In this step, configure an extended ACL to block access to a Web server. Before  proceeding, issue the no access-list 50 and no ip access-group 50 commands on  the Vista router to remove the ACL configured previously.

First, configure both SanJose1 and SanJose2 to act as Web servers, by using the ip  http server command, as shown in the following:

SanJose1(config)#ip http server

SanJose2(config)#ip http server

From the workstation at 192.168.3.2, use a Web browser to view both Web servers on the router  at 10.0.0.1 and 10.0.0.2. The Web login requires that the enable secret password for the router be  entered as the password.

After verifying Web connectivity between the workstation and the routers, proceed to Step 6.

Step 6

On the Vista router, enter the following commands:

Vista(config)#access-list 101 deny tcp 192.168.3.0 0.0.0.255 10.0.0.0  0.0.0.255 eq www

Vista(config)#access-list 101 deny tcp 192.168.3.0 0.0.0.255 any eq ftp Vista(config)#access-list 101 permit ip any any

Vista(config)#interface fastethernet 0/0

Vista(config-if)#ip access-group 101 in

From the workstation at 192.168.3.2, again attempt to view the Web servers at 10.0.0.1  and 10.0.0.2. Both attempts should fail.

Next, browse SanJose1 at 192.168.1.2.